Document: Exported X-Ray Geometry CSV Specification

C-Arm Simulator → DiffDRR Integration

1. Purpose of This Document

This document explains the structure and meaning of the CSV file exported at the moment of X-ray exposure in the simulator.

Each CSV row represents one complete X-ray projection geometry snapshot.

The exported values are sufficient to reconstruct the projection in DiffDRR or any other perspective X-ray renderer without using simulator slider values.

2. Coordinate System Definition

All geometric values in the CSV are expressed in the Simulation World Coordinate System:

- Right-handed coordinate system

- Units: meters

- +Y axis: Up (vertical)

- X–Z plane: Floor plane

- Origin: Arbitrary room reference

- All positions are rigidly defined in this same frame

No values are relative. All are absolute world coordinates.

3. What One CSV Row Represents

Each row defines:

- A single X-ray source position

- A single detector plane in 3D space

- A perspective projection geometry

- Projection intrinsics (field of view + resolution)

This is equivalent to a calibrated pinhole camera model where:

- Camera center = X-ray focal spot

- Image plane = detector plane

- Rays = diverging from source through detector pixels

4. CSV Fields and Their Meaning

4.1 Source Position (Camera Center)

Fields:

- src_pos_world_x

- src_pos_world_y

- src_pos_world_z

Meaning:

These values define the 3D world position of the X-ray focal spot.

All projection rays originate from this point.

Imaging Effect:

- Changing source position changes magnification and perspective.

- This is the most critical geometric parameter.

4.2 Detector Plane Definition

A. Detector Center

Fields:

- det_center_world_x

- det_center_world_y

- det_center_world_z

Meaning:
This is the 3D world position of the center of the detector plane.

It defines where the image plane exists in space.

B. Detector Orientation (Quaternion)

Fields:

- det_quat_world_qx

- det_quat_world_qy

- det_quat_world_qz

- det_quat_world_qw

Meaning:
This quaternion defines the rotation of the detector plane in 3D space.

From this orientation, one can derive:

- Detector horizontal axis (u)

- Detector vertical axis (v)

- Detector normal axis (n)

The detector normal should point approximately toward the source.

Together, detector center + quaternion fully define the image plane.

4.3 Source-to-Detector Distance

Field:

- sid_m

Meaning:

Euclidean distance between source and detector center.

Physically known as SDD (Source-Detector Distance).

Imaging Effect:

Controls projection magnification.

Typical clinical values:
~1.0–1.2 meters.

4.4 Beam Direction

Fields:

- beam_dir_x

- beam_dir_y

- beam_dir_z

Meaning:

Normalized vector from source to detector center.

Represents the central beam axis.

Should equal:

normalize(detector_center − source_position)

Used for validation and reconstruction sanity checks.

4.5 Isocenter

Fields:

- iso_world_x

- iso_world_y

- iso_world_z

Meaning:

The defined mechanical isocenter of the C-arm.

The system’s rotations are defined around this point.

In ideal geometry:

- The central beam passes through this point.

- It lies approximately midway between source and detector.

4.6 ISO Alignment Metrics

A. ISO-to-Ray Distance

Field:

- iso_ray_dist_m

Meaning:
Perpendicular distance from isocenter to the infinite central ray.

If near zero:

- Beam is properly centered.

If large:

- Beam is offset.

B. ISO Projection Parameter

Field:

- t_iso

Meaning:
Position of isocenter along the beam axis.

Interpretation:

- 0 → ISO at source

- 1 → ISO at detector

- 0.5 → ISO midway between

Ideal configuration:
t_iso ≈ 0.5

4.7 Projection Intrinsics

Fields:

- fov_deg

- img_width_px

- img_height_px

- near_m

- far_m

Meaning:

These define the projection frustum.

- Field of view defines angular spread.

- Image width/height define pixel resolution.

- Near/far define valid ray integration bounds.

From these, a renderer derives focal length and intrinsic matrix.

4.8 Control State (Optional)

Fields:

- lift

- cart_x

- cart_z

- orbital_slide_rad

- wig_wag_rad

- column_rot_rad

These represent simulator controls at exposure time.

They are included for reproducibility but are not required to reconstruct the DRR.

5. How to Reconstruct the Projection from CSV

Given one CSV row:

- Convert positions from meters to millimeters (if required by renderer).

- Use source position as camera center.

- Construct detector plane using:

  - detector center

  - quaternion-derived axes

- Derive focal length from FOV and image height.

- For each pixel:

  - Generate ray from source through detector plane

  - Integrate attenuation through CT volume.

No additional simulator information is needed.

6. Mathematical Interpretation

The CSV defines a rigid perspective projection:

For any CT point X:

X_cam = R * (X − C)

Where:

- C = source position

- R = rotation derived from detector orientation

Projection is then defined by intrinsic matrix K derived from FOV.

The DRR pixel value equals the line integral of attenuation along each ray.

7. Validation Rules

For correct geometry:

- sid_m > 0

- beam_dir is normalized

- iso_ray_dist_m ≈ 0 (for centered beam)

- 0 ≤ t_iso ≤ 1

- Detector normal aligns with beam direction

8. Important Notes

- All values are absolute.

- No Euler angles are used (to avoid ambiguity).

- No simulator assumptions are required to interpret the file.

- The CSV fully encodes the X-ray imaging configuration.

9. Summary

Each exported CSV row defines:

A complete calibrated X-ray imaging geometry in 3D space, sufficient to render a DRR from a CT volume without access to the simulator.